BEECH 99

NORMAL PROCEDURES

Use of this checklist is predicated upon pilot’s complete understanding of the “normal procedures” checklist located in Section II of the AFM.

TWO PILOT CREW PROCEDURES

 Non Flying Pilot reads checklist

 Flying Pilot responds

# BEFORE STARTING ENGINE

1.	Exterior Inspection – COMPLETED

2.	Cabin Door – LOCKED

3.	Emergency Exits – SECURE & UNLOCKED

4.	Manifest – COMPLETE/ON BOARD

5.	Control Locks – REMOVED

6.	Seat Belts – SECURE

7.	Brakes – TEST

8.	Emergency Static Air Valve – NORMAL

9.	All Switches – OFF

10.	Landing Gear Handle – DOWN

11.	 Power Levers – IDLE

12.	 Propeller Controls – FULL FORWARD

13.	 Condition Levers – CUT-OFF

14.	 Cabin Temp Mode & Blower – OFF

15.	 Circuit Breakers – IN

16.	 Battery Switch – ON

17.	 Landing Gear Handle Lights – TEST

18.	 Annunciator Panel/Warning Lights – TEST

19.	 Fuel Firewall Shut Offs – TEST (primary & secondary pumps)

20.	 Crossfeed Test – CLOSED

21.	 Fire Detectors – CHECK

- Fuel Quantity – SUFFICIENT

- Pax Briefing – Completed (or to go).

## ENGINE START

# Right Engine

1.	Right Boost Pump – ON

2.	Right Ignition & Start Switch – ON

3.	Right Condition Lever – LOW IDLE (after N1 rpm stabilizes for 5 seconds; 12% minimum)

4.	ITT & N1 – MONITOR (1090ºC Max)

5.	Right Ignition & Start Switch – OFF (at 50% N1 or above)

6.	Right N1 Speed – ADJUST to a minimum of 15% above idle

7.	Right Generator – ON

8.	Right Oil Pressure – Prop out of feather

# Left Engine

Same procedure as for Right Engine.

Note:  Either engine may be started first at Pilot’s Discretion.

## AFTER STARTING & TAXI

- Inverters – BOTH CHECKED then select Inverter to be used

- Right & Left Oil Pressure – CHECK by gauge pressure

- Radios – ON

- Cabin Temperature & Mode – AS REQUIRED

- Gyro & Deice Pressure – CHECK

- Voltage & Loadmeters – CHECK

- Cabin Sign – ON

- Lights – AS REQUIRED

- Annunciator Lights – OUT

- Instruments – CHECK

- Brakes – CHECK

- Block Timer – STARTED.

## BEFORE TAKEOFF

1.	Pitch Trim Indicator – AGREES WITH STABILIZER POSITION***

2.	Pitch Trim System

a.	Main System – CHECK

b.	Stand-by System – CHECK

c.	Out of Trim Warning System – CHECK

3.	Pitch Trim Indicator – SET FOR TAKEOFF (GREEN ARC)

4.	Trim Tabs – SET

5.	Engine Control Friction Locks – ADJUST

6.	Flaps – CHECK & SET FOR Takeoff

BEFORE TAKEOFF cont.

- Flight Controls – CHECK

- Overspeed Governors – TEST***

- Primary Governors – EXERCISE AT 1900 RPM***

- Engine Ice Protection Controls – PULL:  check torque drop; PUSH:  regain original torque***

- Autofeather – ON

- Secondary Low Pitch Stops (if installed) – TEST***

- Propeller Feathering (manual) – CHECK at LOW IDLE

- Radios & Radar – CHECK

- Oil Temperature – CHECK

- Ice Protection – AS REQUIRED

*** May be omitted for quick turn-around at pilot’s discretion.

## TAKEOFF

Monitor ITT & Engine Torque.  Check Auto feather armed lights ON.  Increasing airspeed will cause torque & ITT to increase.  Observe Landing Gear & Flap Operating Airspeed Limits.  Normal Rotation Speed = 90 Knots.  Props set at 2200 RPM.

- Line Items Complete

- Flight Timer - STARTED

### CLIMB

- Autofeather Switch – OFF

- Climb Power – SET

- Propeller RPM – 2,000 RPM

- Engine Instruments – MONITOR

- Cabin Sign – AS REQUIRE

- Landing Lights – OFF (above 3000’ agl)

## CRUISE

1.	Cruise Power – SET

2.	Engine Instruments – MONITOR

- Fuel Quantity – MONITOR

- Taxi & Takeoff Time - RECORDED

## DESCENT

1.	Power – AS REQUIRED

2.	Altimeter – SET

3.	Cabin Sign – AS REQUIRED

4.	Windshield Anti-Ice – AS REQUIRED

5.	Approach Procedure - Reviewed

## LANDING

1.	Heater – OFF

2.	Cabin Sign – ON

3.	Auto Feather Switch – ARM

4.	Flaps – AS REQUIRED

5.	Landing Gear – DOWN

6.	Landing & Taxi Lights – AS REQUIRED

- Power Levels – BETA RANGE  - as reqd.

- Flight Timer – STOPPED after touchdown

BALKED LANDING

1.	 Power – TAKE-OFF (1315 ft-lbs. or ITT 725ºC at 2200 RPM)

2.	Airspeed – BALKED LANDING CLIMB SPEED (When clear of obstacles ESTABLISH 100 KNOTS.)

3.	Flaps -  UP

4.	Landing Gear – UP

# AFTER LANDING

- Props – FORWARD

- Flaps – UP

- Transponder - OFF

- Landing & Taxi Lights – AS REQUIRED

- Ice Protection – OFF

- Electrical Load – OBSERVE LIMITS

- Trim – SET TO ZERO

- Auto Feather Switch – OFF

## ENGINE SHUTDOWN & SECURING

- Parking Brake – SET

- Radios – OFF

- Cabin Vent Blower & Mode Control – OFF

- Inverter – OFF

- Light Switches – OFF

- ITT – BELOW 610º FOR ONE MINUTE

- Propellers – FEATHERED

- Condition Levers – CUT-OFF

- Boost Pumps – OFF

- Battery & Generator Switches – OFF (Below 15% N1)

- Flight and Block Time – RECORDED.